/*! \page index GNU 68HC11 and 68HC12 Examples

This is a collection of examples of 68HC11/68HC12 programs in C

and assembly to show how to use the GNU development chain.

These examples can be built for several boards and porting them to

another board should be easy.

They come with a framework to build them.  This framework

consists of:

<ul>

<li>A set of includes that try to hide the details of

the board implementation and the details of the micro-controller.

<li>A utility library which contains useful functions for

the examples (printing on the serial line for example),

<li>A simple make configuration environment to automate

and help in building the examples.

<li>A documentation framework based on \b doxygen.

</ul>

Examples and the library are presented more into details

in the following sections:

<ul>

<li>Section \ref build

<li>Section \ref porting

<li>Section \ref utilLibrary

<li>Section \ref bootExamples

<li>Section \ref benchExamples

<li>Section \ref miscExamples

</ul>

*/

/*! \page build Build the examples

The examples are ported on several boards. You can select the board

on the command line with \b make or by changing the \em board.def

file.

The following boards are supported:

<TABLE BORDER="1">

<TR>

<TD WIDTH="30%" VALIGN="TOP">m68hc11-32k</TD>

<TD>68HC11 generic board with 32K ram and 32K rom

<p>

<TABLE BORDER="0">

<TR>

<TD>Data</TD>

<TD>0x1040..0x7fff</TD>

</TR>

<TR>

<TD>Stack</TD>

<TD>0x7fff</TD>

</TR>

<TR>

<TD>Text</TD>

<TD>0x8000..0xffff</TD>

</TR>

</TABLE>

<p>

Prefer this board if you want to use the simulator.

</TD>

</TR>

<TR>

<TD VALIGN="TOP">m68hc11-cme11</TD>

<TD>Axiom Manufacturing CME11 board with Buffalo monitor

<p>

<TABLE BORDER="0">

<TR>

<TD>Data</TD>

<TD>0x1040..0x1fff</TD>

</TR>

<TR>

<TD>Stack</TD>

<TD>0x1fff</TD>

</TR>

<TR>

<TD>Text</TD>

<TD>0x2000..0x7fff</TD>

</TR>

</TABLE>

</TD>

</TR>

<TR>

<TD VALIGN="TOP">m68hc11-ebcs</TD>

<TD>68HC11 board with 8K ram (SCz board)

<p>

<TABLE BORDER="0">

<TR>

<TD>Data</TD>

<TD>0xE000..0xE1ff</TD>

</TR>

<TR>

<TD>Stack</TD>

<TD>0x0ff</TD>

</TR>

<TR>

<TD>Text</TD>

<TD>0xE200..0xFfff</TD>

</TR>

</TABLE>

</TD>

</TR>

<TR>

<TD VALIGN="TOP">m68hc12-axcmd12</TD>

<TD>Axiom Manufacturing CMD12 board with DBug monitor

<p>

<TABLE BORDER="0">

<TR>

<TD>Data</TD>

<TD>0x2000..0x2fff</TD>

</TR>

<TR>

<TD>Stack</TD>

<TD>0x3000</TD>

</TR>

<TR>

<TD>Text</TD>

<TD>0x3000..0x8000</TD>

</TR>

</TABLE>

</TD>

</TR>

</TABLE>

The safest way is to change the file \em config/board.def and

update the \c BOARD definition by changing the line:

<pre>

BOARD=<i>board-name</i>

</pre>

where \em board-name is the name of the board picked from the

table above.

If you prefer not modify this file, you pass the

\c BOARD=board-name option to \b make

but you must be very careful that the \em lib directory is

compiled for the same board.

Before launching \b make you should make sure the GNU Binutils

and Gcc executables are in your path. Then, proceed with the following

commands:

<pre>

make

</pre>

or

<pre>

make BOARD=<i>board-name</i>

</pre>

This will build the examples in all the sub-directories.

You can then check the programs by typing:

<pre>

m6811-elf-run simple/simple.elf

m6811-elf-run calculator/calc.elf

m6811-elf-run trap/trap.elf

</pre>

*/

/*! \page porting Porting the examples.

The easiest way to adapt the examples for your board is probably

to copy the \c m68hc11-32k directory to another directory and update

the \c memory.x file and optionally the \c make.defs file to define the

characteristics of your board.

Have also a look at \c lib/exit.c to see whether the \c exit

implementation works for your board.

*/

/*! \page utilLibrary Utility Library

To increase the portability of programs among several boards,

the utility library provides a set of includes and functions

that hide these differences.  It also provides a set of commonly

used functions such as benchmarking, and other general purpose

operations.

<p>

To use this library, you must:

<ul>

<li>Add a <b>-I<i>install-dir</i>/include</b> directive to gcc and cpp,

<li>Link with <b><i>install-dir</i>/lib/libutil.a</b>.

</ul>

The utility library provides the following modules:

<ul>

<li>\ref locks

<li>\ref sio

<li>\ref interrupts

<li>\ref imath

<li>\ref benchs

</ul>

*/

/*! \page bootExamples Bootstrap Examples

These examples are very small and they are designed to fit in less

than 256 bytes (less than 240 to leave room for the stack).  They

represent typical programs that can be uploaded during the 68HC11

bootstrap mode.

<ul>

<li>\ref blink

<li>\ref booter

<li>\ref checker

<li>\ref dumper

</ul>

In general to compile and link these programs, you will use the

following %command:

<pre>

m6811-elf-gcc -o file.elf -nostdlib -nostartfiles \

-Wl,-defsym,_io_ports=0x1000 \

-Wl,-defsym,_.tmp=0x0 -Wl,-defsym,_.z=0x2 \

-Wl,-m,m68hc11elfb \

-mshort -Os -fomit-frame-pointer -msoft-reg-count=0 \

file.c

</pre>

In this cryptic and long %command, the following options are

used by the compiler:

<TABLE>

<TR>

<TD>

-mshort

</TD>

<TD>

This option tells gcc to use 16-bit integers.

</TD>

</TR>

<TR>

<TD>

-Os

</TD>

<TD>

This option tells gcc to optimize for size.

</TD>

</TR>

<TR>

<TD>

-fomit-frame-pointer

</TD>

<TD>

This option tells gcc to remove the frame pointer.

</TD>

</TR>

<TR>

<TD>

-msoft-reg-count=0

</TD>

<TD>

This option tells gcc to avoid to use soft registers

(except _.tmp and _.z which are sometimes necessary)

</TD>

</TR>

</TABLE>

And the following options are used by the linker:

<TABLE>

<TR>

<TD>

-nostdlib

</TD>

<TD>

The standard libraries are not passed to the linker.

</TD>

</TR>

<TR>

<TD>

-nostartfiles

</TD>

<TD>

The standard startup files are not used during the link.

</TD>

</TR>

<TR>

<TD>

-defsym _io_ports=0x1000

</TD>

<TD>

The linker will define the symbol \c _io_ports and

assign it the value \c 0x1000.  This symbol is used

by the examples to access the 68HC11 registers.

</TD>

</TR>

<TR>

<TD>

-defsym _.tmp=0x0<br>

-defsym _.z=0x0

</TD>

<TD>

This will define the symbol \c _.tmp used by gcc.

</TD>

</TR>

</TABLE>

*/

/*! \page miscExamples Other Examples

<ul>

<li>\ref calculator

<li>\ref primes

<li>\ref trap

<li>\ref timer

<li>\ref analyzer

</ul>

*/

/*! \page benchExamples Benchmark Examples

The benchmark programs are intended to measure the efficiency of

the generated code.

<ul>

<li>\ref bench-basic

<li>\ref bench-string

<li>\ref bench-float

<li>\ref bench-misc

</ul>

*/