Keypad to LCD routines.

General: These routines provide sample source code to scan the keypad

and send the key or function to the LCD dispaly module. All ".asm"

type source can be assembled with the AS11.EXE assembler and are

located to run under the buffalo monitor program.

These codes are located a hex $2000 in the memory map. JP3 must be

installed on 8K version boards for proper operation.

Source files:

Keylcd-e.asm	: Source file for the CME11 and CMM11 keypad port.

Keylcd-d.asm	: Source file for the CMD11 keypad port.

Key_lcd.c	: Small-C 11 source file for CMD11 type keypad port.

Keylcd-e.bas	: Basic11 source file for CME and CMM type keypad port.

Keypad-d.bas	: Basic11 source file for CMD11 type keypad port, this

routine prints key to the terminal instead of

LCD.

Use:

Keys 0-9,A,B,C,D,# will display on LCD, key * will clear LCD display.

To Load:

At the Buffalo prompt, enter 'LOAD T' then press return.

Use the terminal program to do an ASCII DOWNLOAD of file 'KEYLCD- .S19'.

At the Buffalo prompt, enter 'GO 2000' then press return.

Program is now operating.

######################