| Address | Name | Description |
| --- | --- | --- |
| $E000 | BUFFALO | Restart Buffalo Monitor (jmp) |
| $1404 | LED | LED’s (PB0-7 on Fox 11 Boards - Write Only) |
| $1403 | Switch | Dip Switch (PC0-7 on Fox 11 Boards - Read Only) |

| JSR Address | Name | Description |
| --- | --- | --- |
| $FFB2 | OUTLHLF | Convert left nibble of accumulator A contents to ASCII and output to terminal port. |
| $FFB5 | OUTRHLF | Convert right nibble of accumulator A contents to ASCII and output to terminal port. |
| $FFB8 | OUTA | Output accumulator A ASCII character. |
| $FFBB | OUTlBYT | Convert binary byte at address in index register X to two ASCII characters and output. Returns address in index register X pointing to next byte. |
| $FFBE | OUTlBSP | Convert binary byte at address in index register X to two ASCII characters and output followed by a space. Returns address in index register |
| $FFCl | OUT2BSP | Convert two consecutive binary bytes starting at address in index register X to four ASCII characters and output followed by a space. Returns address in index register X pointing to next byte. |
| $FFC4 | OUTCRLF | Output ASCII carriage return followed by a line feed. |
| $FFC7 | OUTSTRG | Output string of ASCII bytes pointed to by address in index register X until character is EOT - end of transmission ($04). |
| $FFCA | OUTSTRGO | Same as OUTSTRG except leading carriage return and line feed is skipped. |
| $FFCD | INCHAR | Input ASCII character to accumulator A and echo back. This routine loops until character is actually received. |
| (Routines below are not standard Buffalo routines (FOX11 Specific) |  |  |
| $FF70 | LCD_INI | initialize a 16x2 LCD display module |
| $FF73 | LCD_LINE1 | Output string of 16 ASCII bytes pointed to by address in index register X to the first line of a 16X2 LCD display module. |
| $FF76 | LCD_LINE2 | Output string of 16 ASCII bytes pointed to by address in index register X to the second line of a 16X2 LCD display module. |